FOUNDATION DESIGN SCHEDULE

Project: Multi-Storey Residential Complex, Sector 14, Gurugram, Haryana

Doc Ref: FDS-2024-GGN-047 | Rev: A | Date: May 2024 | IS:456-2000, IS:2911-2010, IS:6403-1981

# 1. DESIGN BASIS

1.1 Geotechnical Parameters: Adopted from Site Investigation Report SIR-2024-GGN-047. Safe Bearing Capacity (SBC) adopted as per borehole locations. Seismic Zone IV (IS:1893-2016). Importance Factor = 1.5. Response Reduction = 5.0 (SMRF).

1.2 Load Cases: Dead Load (DL), Live Load (LL), Seismic Load (EQ), Wind Load (WL) per IS:875. Load combinations per IS:456-2000 Table 18. Factored loads (ULS) per IS:456. Foundation design at serviceability (SLS) for soil pressure check.

1.3 Foundation Type Selection Criteria: Column axial load < 1500 kN — Isolated Footing. Column axial load 1500-2500 kN — Large Isolated Footing or combined footing. Column axial load > 2500 kN — Bored Pile Foundation. Core walls and shear walls — Raft Foundation.

# 2. ISOLATED FOOTING SCHEDULE

## 2.1 Design Parameters Adopted

| Ftg ID | Col Grid | Axial Load DL+LL (kN) | Factored Load (kN) | SBC Adopted (kN/m2) | BH Ref | Footing Size (mm) | Depth (mm) | Conc Grade | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FTG-A01 | A1 | 480 | 672 | 127 | BH-01/04 | 2000x2000 | 500 | M30 | Edge column |
| FTG-A02 | A2 | 510 | 714 | 127 | BH-01/04 | 2100x2100 | 500 | M30 | Edge column |
| FTG-A03 | A3 | 490 | 686 | 127 | BH-01/04 | 2000x2000 | 500 | M30 | Edge column |
| FTG-A04 | A4 | 520 | 728 | 127 | BH-01/04 | 2100x2100 | 550 | M30 | Corner column |
| FTG-A05 | A5 | 495 | 693 | 127 | BH-04 | 2000x2000 | 500 | M30 | Interior |
| FTG-A06 | A6 | 505 | 707 | 127 | BH-04 | 2100x2100 | 500 | M30 | Interior |
| FTG-B01 | B1 | 680 | 952 | 120 | BH-02/05 | 2700x2700 | 600 | M30 | Central zone |
| FTG-B02 | B2 | 720 | 1008 | 120 | BH-02/05 | 2800x2800 | 650 | M30 | Central zone |
| FTG-B03 | B3 | 710 | 994 | 120 | BH-05 | 2750x2750 | 620 | M30 | Central zone |
| FTG-B04 | B4 | 695 | 973 | 120 | BH-05 | 2700x2700 | 600 | M30 | Central zone |
| FTG-B05 | B5 | 740 | 1036 | 120 | BH-05 | 2900x2900 | 680 | M30 | Central zone |
| FTG-B06 | B6 | 700 | 980 | 120 | BH-02/05 | 2750x2750 | 620 | M30 | Central zone |
| FTG-B07 | B7 | 685 | 959 | 120 | BH-02 | 2700x2700 | 600 | M30 | Central zone |
| FTG-B08 | B8 | 715 | 1001 | 120 | BH-05 | 2800x2800 | 650 | M30 | Central zone |
| FTG-C01 | C1 — HIGH LOAD | 1850 | 2590 | 120 | BH-05 | 4700x4700 | 900 | M30 | INCONSISTENCY: SBC used = 120 correct but footing size is under-designed; required size = sqrt(2590/120) = 4.65m OK; however design doc states SBC=150 — WRONG |
| FTG-C02 | C2 — HIGH LOAD | 1920 | 2688 | 150 | BH-05 | 4230x4230 | 900 | M30 | INCONSISTENCY: SBC adopted = 150 kN/m2 but BH-05 SBC = 120 kN/m2 at 2.5m (SIR Table 4.1). Size based on WRONG SBC. Correct size = sqrt(2688/120) = 4.73m, not 4.23m. |
| FTG-C03 | C3 | 820 | 1148 | 125 | BH-03/06 | 3100x3100 | 700 | M30 | Zone C OK |
| FTG-C04 | C4 | 810 | 1134 | 125 | BH-06 | 3050x3050 | 700 | M30 | Zone C OK |
| FTG-D01 | D1 — CORNER | 1100 | 1540 | 127 | BH-01 | 3500x3500 | 800 | M30 | Heavy corner col |
| FTG-D02 | D2 — CORNER | 1080 | 1512 | 127 | BH-04 | 3450x3450 | 800 | M30 | Heavy corner col |
| FTG-D03 | D3 — CORNER | 1090 | 1526 | 125 | BH-03 | 3500x3500 | 800 | M30 | Heavy corner col |
| FTG-D04 | D4 — CORNER | 1070 | 1498 | 125 | BH-06 | 3450x3450 | 800 | M30 | Heavy corner col |

CRITICAL INCONSISTENCY: FTG-C02 adopts SBC = 150 kN/m2. Site Investigation Report SIR-2024-GGN-047 Section 4.1 records SBC at BH-05 = 120 kN/m2 at 2.5m founding depth. Use of 150 kN/m2 is INCORRECT and results in an under-designed footing. Required plan area = 2688/120 = 22.4 m2 (4.73m x 4.73m). Designed area = 4.23m x 4.23m = 17.89 m2 — 21% DEFICIENT.

# 3. PILE FOUNDATION SCHEDULE

## 3.1 Pile Design Parameters

Pile type: Bored cast-in-situ piles. Diameter: 600mm. Concrete grade: M30. Reinforcement: Fe500D full-length cage, 6 nos T20 longitudinal bars + T10 @ 150mm helical ties. Pile toe depth: 15m bgl. Safe pile capacity: 1800 kN per pile (adopted).

INCONSISTENCY: This document states safe pile capacity = 1800 kN for 600mm dia pile at 15m depth. Site Investigation Report SIR-2024-GGN-047 Section 5 (Pile Design Parameters table) records safe capacity for 600mm dia pile at 15m depth = 1480 kN (interpolated between 14m=1250 kN and 16m=1680 kN per IS:2911). Value of 1800 kN is OVERSTATED by 320 kN (22%). Pile group must be re-designed using 1480 kN per pile.

## 3.2 Pile Group Schedule

| Pile Cap ID | Col Grid | Axial Load DL+LL (kN) | Factored Load (kN) | No. of Piles (Adopted) | Pile Cap. Used (kN) | Total Cap. (kN) | Utilization Ratio | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PC-01 | E1 | 2600 | 3640 | 2 | 1800 | 3600 | 1.011 | INCONSISTENCY: 2 piles with 1800kN = 3600kN < 3640kN load — OVERSTRESSED. At correct 1480kN: cap=2960kN < 3640kN — requires 3 piles. |
| PC-02 | E2 | 2750 | 3850 | 3 | 1800 | 5400 | 0.713 | INCONSISTENCY: Uses wrong pile capacity. At 1480kN: cap=4440kN > 3850kN. 3 piles adequate but utilization changes to 0.867. |
| PC-03 | E3 | 2580 | 3612 | 2 | 1800 | 3600 | 1.003 | INCONSISTENCY: Same as PC-01. Overstressed. Requires 3 piles at true capacity. |
| PC-04 | F1 | 3200 | 4480 | 3 | 1800 | 5400 | 0.830 | At correct 1480kN: 3 piles = 4440kN < 4480kN — STILL DEFICIENT. Requires 4 piles. |
| PC-05 | F2 | 2900 | 4060 | 3 | 1800 | 5400 | 0.752 | At correct 1480kN: 3 piles = 4440kN > 4060kN. OK with 3 piles. |
| PC-06 | F3 | 2650 | 3710 | 3 | 1800 | 5400 | 0.687 | At correct 1480kN: 3 piles = 4440kN > 3710kN. OK with 3 piles. |

## 3.3 Pile Cap Concrete Schedule

| Pile Cap | Size (mm) | Depth (mm) | Conc Grade | Bot Steel | Top Steel | Links | Cover (mm) | Volume (m3) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PC-01 | 2000x2000 | 900 | M30 | 12T20@150 | 8T16@200 | T10@200 | 75 | 3.60 |
| PC-02 | 2800x2800 | 1000 | M30 | 14T20@150 | 10T16@200 | T10@200 | 75 | 7.84 |
| PC-03 | 2000x2000 | 900 | M30 | 12T20@150 | 8T16@200 | T10@200 | 75 | 3.60 |
| PC-04 | 2800x2800 | 1100 | M30 | 16T20@150 | 12T16@200 | T10@150 | 75 | 8.62 |
| PC-05 | 2800x2800 | 1000 | M30 | 14T20@150 | 10T16@200 | T10@200 | 75 | 7.84 |
| PC-06 | 2800x2800 | 1000 | M30 | 14T20@150 | 10T16@200 | T10@200 | 75 | 7.84 |

## 3.4 Tie Beams at Pile Cap Level

Tie beams provided between all pile caps as per IS:2911. Size: 300x600mm, M30 concrete, Fe500 steel. Reinforcement: 4T16 top and bottom + T8@200mm links. Tie beam forces: 10% of axial column load per IS:13920.

# 4. RAFT FOUNDATION DESIGN — CORE WALL ZONE

## 4.1 Raft Parameters

Location: Core wall zone (Lift core + staircase). Plan area: 18.0m x 12.0m = 216 m2. Founding depth: 3.0m bgl. Concrete grade: M35 (severe exposure per IS:456 Table 5). Steel: Fe500D.

MATERIAL INCONSISTENCY: construction_cost_estimate.pdf Item A2.06 records M30 for raft foundation. This design specifies M35 (required for severe underground exposure per IS:456-2000 Table 5 and site_investigation_report.pdf Section 8). The correct grade is M35 as specified here.

## 4.2 Raft Soil Pressure Check

| Load Combination | Total Load (kN) | Raft Area (m2) | Net SBC Used (kN/m2) | Actual Pressure (kN/m2) | Safety Factor | Status |
| --- | --- | --- | --- | --- | --- | --- |
| DL+LL (SLS) | 18420 | 216 | 100 | 85.3 | 1.17 | OK |
| DL+LL+EQ (SLS) | 18420 + 2100 EQ | 216 | 100 | 95.0 | 1.05 | MARGINAL |
| 1.5(DL+LL) ULS | 27630 | 216 | N/A (ULS Flexure) | — | — | Flexure governs |

## 4.3 Raft Reinforcement Schedule

| Zone | Direction | Bottom Steel | Top Steel | Raft Thickness (mm) | Cover (mm) | IS Reference | OK? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Core wall zone | X-dir | T20@150 | T16@200 | 1000 | 75 | IS:456-2000 D-2 | OK |
| Core wall zone | Y-dir | T20@150 | T16@200 | 1000 | 75 | IS:456-2000 D-2 | OK |
| Column strips | X-dir | T25@125 | T20@150 | 1000 | 75 | IS:456-2000 D-2 | OK |
| Column strips | Y-dir | T25@125 | T20@150 | 1000 | 75 | IS:456-2000 D-2 | OK |
| Middle strips | X-dir | T16@200 | T12@250 | 1000 | 75 | IS:456-2000 D-2 | OK |
| Middle strips | Y-dir | T16@200 | T12@250 | 1000 | 75 | IS:456-2000 D-2 | OK |

# 5. MATERIAL COMPLIANCE SUMMARY

## 5.1 Concrete Grades Specified in This Document

| Foundation Element | Grade Specified Here | Grade in SIR | Grade in CCE | Grade in MatSpec | Consistent? | True Grade |
| --- | --- | --- | --- | --- | --- | --- |
| Isolated Footings A,B,C,D | M30 | M30 | M30/M25(FTG-C) | M30 (MAT-013) | PARTIAL | M30 |
| Pile Caps | M30 | M30 | M25 (CCE A2.05) | M30 (MAT-075) | NO | M30 |
| Raft Foundation | M35 | M35 | M30 (CCE A2.06) | M35 (MAT-047) | NO | M35 |
| Bored Piles | M30 | M30 | M30 (CCE A4.01) | M30 (MAT-077) | YES | M30 |
| Tie Beams | M30 | Not specified | M30 (CCE A3.04) | M30 (MAT-052) | YES | M30 |

## 5.2 Known Inconsistencies — Record of Conflicts

| # | Element | This Document | Conflict Source | Conflict Value | Resolution (True Value from Evidence) |
| --- | --- | --- | --- | --- | --- |
| 1 | FTG-C02 SBC | 150 kN/m2 | SIR BH-05 Table 4.1 | 120 kN/m2 | True SBC = 120 kN/m2. Footing under-designed by 21%. |
| 2 | Pile safe capacity 600mm@15m | 1800 kN | SIR Section 5 | 1480 kN | True value = 1480 kN per IS:2911 calc. PC-01,03 overstressed, PC-04 deficient. |
| 3 | Raft concrete grade | M35 | CCE Item A2.06 | M30 | True grade = M35 per IS:456 Table 5 severe exposure. CCE is wrong. |
| 4 | Pile cap concrete grade | M30 | CCE Items A2.05 | M25 | True grade = M30 per SIR Sec 8 and MatSpec MAT-075. |
| 5 | PC-01 pile count | 2 piles | Corrected per SIR pile table | 3 piles required | 2 piles at 1480kN = 2960kN < 3640kN load. 3 piles required. |

Prepared by: Structural Design Division  |  Checked by: Senior Structural Engineer  |  Approved by: Technical Director

All designs per IS:456-2000, IS:2911-2010, IS:1893-2016, IS:13920-2016. Site Investigation Report SIR-2024-GGN-047 is the primary geotechnical reference.